#include <Arduino.h>

#include <ESP32SPISlave.h>

ESP32SPISlave slave;

static constexpr size_t BUFFER_SIZE = 8;

uint8_t txBuffer[BUFFER_SIZE] = {

0xA5, 0x5A, 0x11, 0x22,

0x33, 0x44, 0x55, 0xAA

};

uint8_t rxBuffer[BUFFER_SIZE] = {0};

uint8_t expectedData[BUFFER_SIZE] = {

0x10, 0x20, 0x30, 0x40,

0x50, 0x60, 0x70, 0x80

};

void setup() {

Serial.begin(115200);

slave.setDataMode(SPI_MODE0);

// SPI pins: SCK=18, MISO=19, MOSI=23, CS=5

if (!slave.begin(VSPI, 18, 19, 23, 5)) {

Serial.println("SPI initialization failed!");

while (true) delay(1000);

}

Serial.println("ESP32 SPI slave ready");

}

void loop() {

// Wait for one 8-byte transaction.

size_t received = slave.transfer(

txBuffer, rxBuffer, BUFFER_SIZE, 1000

);

if (received == 0) {

return;

}

bool valid = received == BUFFER_SIZE;

Serial.print("Received bytes: ");

for (size_t i = 0; i < received; i++) {

Serial.printf("%02X ", rxBuffer[i]);

if (i >= BUFFER_SIZE || rxBuffer[i] != expectedData[i]) {

valid = false;

}

}

Serial.println();

Serial.println(valid ? "SPI RX: PASS" : "SPI RX: FAIL");

}

We'll use the RP2040 as the SPI controller and the ESP32 as the SPI peripheral. One important difference from UART: the ESP32's standard Arduino SPI library is primarily designed for controller/master operation, so the peripheral implementation needs special handling. We'll start with a small, fixed-length data frame and verify that the received data matches the expected pattern.