from machine import UART, Pin

import time

# UART0: TX = GPIO28, RX = GPIO29

uart = UART(

0,

baudrate=115200,

bits=8,

parity=None,

stop=1,

tx=Pin(28),

rx=Pin(29),

timeout=200

)

print("RP2040 Communication Tester")

time.sleep(2)

uart.read()  # Clear any old received data

uart.write(b"PING\n")

response = b""

start = time.ticks_ms()

while time.ticks_diff(time.ticks_ms(), start) < 2000:

if uart.any():

response += uart.read()

if b"\n" in response:

break

time.sleep_ms(10)

if response.strip() == b"PONG":

print("UART PING/PONG: PASS")

print("Response:", response.decode().strip())

else:

print("UART PING/PONG: FAIL")

print("Received:", repr(response))