from machine import SPI, Pin

import time

# SPI configuration

cs = Pin(17, Pin.OUT, value=1)

spi = SPI(

0,

baudrate=100000,

polarity=0,

phase=0,

bits=8,

firstbit=SPI.MSB,

sck=Pin(18),

mosi=Pin(19),

miso=Pin(16)

)

# Data sent to ESP32 and expected back

tx_data = bytearray([0x10, 0x20, 0x30, 0x40,

0x50, 0x60, 0x70, 0x80])

expected_response = bytes([

0xA5, 0x5A, 0x11, 0x22,

0x33, 0x44, 0x55, 0xAA

])

TOTAL_TESTS = 100

passed = 0

failed = 0

print("===== SPI TEST 2: REPEATED COMMUNICATION =====")

print("Total transactions:", TOTAL_TESTS)

print("Starting test...\n")

# Give the ESP32 time to enter its receive loop

time.sleep(2)

for test_no in range(1, TOTAL_TESTS + 1):

rx_data = bytearray(8)

cs.value(0)

time.sleep_us(10)

spi.write_readinto(tx_data, rx_data)

cs.value(1)

if bytes(rx_data) == expected_response:

passed += 1

print("Test", test_no, ": PASS")

else:

failed += 1

print("Test", test_no, ": FAIL | Received:",

bytes(rx_data).hex(" ").upper())

# Allow the ESP32 slave to prepare for the next transaction

time.sleep_ms(10)

success_rate = (passed / TOTAL_TESTS) * 100

print("\n===== SPI TEST 2 RESULTS =====")

print("Total tests :", TOTAL_TESTS)

print("Passed      :", passed)

print("Failed      :", failed)

print("Success rate: {:.1f}%".format(success_rate))

if failed == 0:

print("OVERALL RESULT: PASS")

else:

print("OVERALL RESULT: FAIL")

We'll perform 100 SPI transactions between your RP2040 (controller) and ESP32 (slave), count PASS/FAIL results, and calculate the success rate.