from machine import UART, Pin

import time

uart = UART(

0,

baudrate=115200,

bits=8,

parity=None,

stop=1,

tx=Pin(28),

rx=Pin(29)

)

def read_line(timeout_ms=1000):

start = time.ticks_ms()

data = b""

while time.ticks_diff(time.ticks_ms(), start) < timeout_ms:

if uart.any():

data += uart.read()

if b"\n" in data:

return data.split(b"\n", 1)[0].strip()

time.sleep_ms(2)

return None

print("RP2040 - Test 3: Repeated Communication")

time.sleep_ms(200)

while uart.any():

uart.read()

total = 100

passed = 0

failed = 0

for i in range(total):

message = ("DATA:%03d" % i).encode()

uart.write(message + b"\n")

response = read_line(1000)

if response == message:

passed += 1

else:

failed += 1

print("FAIL:", i, "Received:", response)

time.sleep_ms(20)

print("\n--- TEST RESULTS ---")

print("Total messages:", total)

print("Passed:", passed)

print("Failed:", failed)

if failed == 0:

print("Test 3: PASS")

else:

print("Test 3: FAIL")

This test sends 100 messages from the RP2040 to the ESP32 and check how many are returned correctly. This helps identify intermittent communication errors.