from machine import SPI, Pin

import time

cs = Pin(17, Pin.OUT, value=1)

spi = SPI(

0,

baudrate=100000,

polarity=0,

phase=0,

bits=8,

firstbit=SPI.MSB,

sck=Pin(18),

mosi=Pin(19),

miso=Pin(16)

)

patterns = [

bytearray([0x00] * 8),

bytearray([0xFF] * 8),

bytearray([0xAA, 0x55, 0xAA, 0x55,

0xAA, 0x55, 0xAA, 0x55]),

bytearray([0x12, 0x34, 0x56, 0x78,

0x9A, 0xBC, 0xDE, 0xF0])

]

names = [

"ALL ZEROS",

"ALL ONES",

"ALTERNATING BITS",

"MIXED DATA"

]

expected_response = bytes([

0xA5, 0x5A, 0x11, 0x22,

0x33, 0x44, 0x55, 0xAA

])

passed = 0

failed = 0

print("===== SPI TEST 3: DATA INTEGRITY =====")

time.sleep(2)

for i in range(len(patterns)):

tx_data = patterns[i]

rx_data = bytearray(8)

print("\nTest", i + 1, "-", names[i])

print("Sending:", tx_data.hex(" ").upper())

cs.value(0)

time.sleep_us(10)

spi.write_readinto(tx_data, rx_data)

cs.value(1)

time.sleep_ms(50)

print("Received:", bytes(rx_data).hex(" ").upper())

if bytes(rx_data) == expected_response:

print("RP2040 response check: PASS")

passed += 1

else:

print("RP2040 response check: FAIL")

failed += 1

print("\n===== SPI TEST 3 RESULTS =====")

print("Transactions:", len(patterns))

print("Passed:", passed)

print("Failed:", failed)

if failed == 0:

print("RP2040 RESPONSE RESULT: PASS")

else:

print("RP2040 RESPONSE RESULT: FAIL")

print("Check ESP32 Serial Monitor for request data integrity.")

In this test, we'll send different hexadecimal data patterns from the RP2040 to the ESP32 and verify that every byte is received correctly.