#include <Arduino.h>

#include <ESP32SPISlave.h>

ESP32SPISlave slave;

static constexpr size_t BUFFER_SIZE = 8;

uint8_t txBuffer[BUFFER_SIZE] = {

0xA5, 0x5A, 0x11, 0x22,

0x33, 0x44, 0x55, 0xAA

};

uint8_t rxBuffer[BUFFER_SIZE] = {0};

const uint8_t patterns[4][BUFFER_SIZE] = {

{0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00},

{0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF, 0xFF},

{0xAA, 0x55, 0xAA, 0x55, 0xAA, 0x55, 0xAA, 0x55},

{0x12, 0x34, 0x56, 0x78, 0x9A, 0xBC, 0xDE, 0xF0}

};

const char* testNames[4] = {

"ALL ZEROS",

"ALL ONES",

"ALTERNATING BITS",

"MIXED DATA"

};

int testIndex = 0;

void setup() {

Serial.begin(115200);

delay(1000);

slave.setDataMode(SPI_MODE0);

if (!slave.begin(VSPI, 18, 19, 23, 5)) {

Serial.println("SPI initialization failed!");

while (true) {

delay(1000);

}

}

Serial.println("ESP32 SPI slave ready");

Serial.println("Waiting for RP2040 data...");

}

void loop() {

size_t received = slave.transfer(

txBuffer, rxBuffer, BUFFER_SIZE, 1000

);

if (received == 0) {

return;

}

bool valid = (received == BUFFER_SIZE);

if (testIndex < 4) {

for (size_t i = 0; i < BUFFER_SIZE; i++) {

if (rxBuffer[i] != patterns[testIndex][i]) {

valid = false;

}

}

Serial.printf("\nTest %d: %s\n",

testIndex + 1, testNames[testIndex]);

Serial.print("Received: ");

for (size_t i = 0; i < received; i++) {

Serial.printf("%02X ", rxBuffer[i]);

}

Serial.println();

Serial.println(valid ? "ESP32 RX: PASS" : "ESP32 RX: FAIL");

testIndex++;

} else {

Serial.println("Extra transaction received");

}

}