from machine import UART, Pin

import time

uart = UART(

0,

baudrate=115200,

bits=8,

parity=None,

stop=1,

tx=Pin(28),

rx=Pin(29)

)

def read_line(timeout_ms=2000):

start = time.ticks_ms()

data = b""

while time.ticks_diff(time.ticks_ms(), start) < timeout_ms:

if uart.any():

data += uart.read()

if b"\n" in data:

return data.split(b"\n", 1)[0].strip()

time.sleep_ms(5)

return None

def checksum8(data):

return sum(data) & 0xFF

def send_test(payload, corrupt=False):

checksum = checksum8(payload)

if corrupt:

checksum = (checksum + 1) & 0xFF

message = payload + b"|" + ("%02X" % checksum).encode()

uart.write(message + b"\n")

response = read_line()

expected = b"FAIL" if corrupt else b"PASS"

result = response == expected

print("Payload:", payload.decode())

print("Checksum sent:", "%02X" % checksum)

print("Response:", response)

print("Test result:", "PASS" if result else "FAIL")

print()

return result

print("RP2040 - Test 4: Checksum Verification")

time.sleep_ms(300)

while uart.any():

uart.read()

passed = 0

failed = 0

if send_test(b"HELLO_ESP32"):

passed += 1

else:

failed += 1

if send_test(b"RP2040_DATA_12345"):

passed += 1

else:

failed += 1

# Deliberately send an incorrect checksum.

# The ESP32 should reject it.

if send_test(b"CORRUPTED_TEST", corrupt=True):

passed += 1

else:

failed += 1

print("--- TEST 4 RESULTS ---")

print("Checks performed:", passed + failed)

print("Passed:", passed)

print("Failed:", failed)

if failed == 0:

print("Test 4: PASS")

else:

print("Test 4: FAIL")

In this test, the RP2040 will send a message with a checksum. The ESP32 will calculate the checksum independently and report whether the message is valid. We'll also deliberately send one corrupted message to verify that the ESP32 detects the error.