from machine import UART, Pin

import time

# UART0: RP2040 GPIO28 TX -> ESP32 GPIO16 RX

#        RP2040 GPIO29 RX <- ESP32 GPIO17 TX

uart = UART(

0,

baudrate=115200,

bits=8,

parity=None,

stop=1,

tx=Pin(28),

rx=Pin(29)

)

NUM_TESTS = 100

TIMEOUT_MS = 500

def checksum8(data):

return sum(data) & 0xFF

def read_line(timeout_ms=TIMEOUT_MS):

start = time.ticks_ms()

data = b""

while time.ticks_diff(time.ticks_ms(), start) < timeout_ms:

if uart.any():

chunk = uart.read()

if chunk:

data += chunk

if b"\n" in data:

line = data.split(b"\n", 1)[0]

return line.strip()

time.sleep_ms(1)

return None

def make_frame(payload):

checksum = checksum8(payload)

return payload + b"|" + ("%02X" % checksum).encode() + b"\n"

def run_latency_test():

# Clear any leftover input before beginning.

while uart.any():

uart.read()

latencies = []

timeouts = 0

invalid_responses = 0

print("\n===== TEST 5: UART LATENCY =====")

print("Baud rate:", 115200)

print("Requests:", NUM_TESTS)

print("--------------------------------")

for sequence in range(1, NUM_TESTS + 1):

seq = ("%04d" % sequence).encode()

payload = b"LATENCY:" + seq

expected_payload = b"ACK:" + seq

frame = make_frame(payload)

# Start timer immediately before transmission.

start = time.ticks_us()

uart.write(frame)

response = read_line()

elapsed_us = time.ticks_diff(time.ticks_us(), start)

if response is None:

timeouts += 1

print(sequence, "- TIMEOUT")

continue

# Validate acknowledgement and its checksum.

parts = response.rsplit(b"|", 1)

if len(parts) != 2:

invalid_responses += 1

print(sequence, "- INVALID RESPONSE:", response)

continue

response_payload, checksum_text = parts

try:

received_checksum = int(checksum_text, 16)

except ValueError:

invalid_responses += 1

print(sequence, "- INVALID CHECKSUM FIELD")

continue

if (response_payload != expected_payload or

checksum8(response_payload) != received_checksum):

invalid_responses += 1

print(sequence, "- INVALID RESPONSE:", response)

continue

latencies.append(elapsed_us)

print(sequence, "- PASS |", elapsed_us, "us")

# Small gap between requests.

time.sleep_ms(5)

print("\n========== RESULTS ==========")

passed = len(latencies)

attempted = NUM_TESTS

print("Total requests:", attempted)

print("Valid responses:", passed)

print("Timeouts:", timeouts)

print("Invalid responses:", invalid_responses)

print("Success rate: %.2f%%" % (passed * 100 / attempted))

if passed > 0:

average_us = sum(latencies) / passed

minimum_us = min(latencies)

maximum_us = max(latencies)

print("Average round-trip latency: %.2f us" % average_us)

print("Minimum round-trip latency:", minimum_us, "us")

print("Maximum round-trip latency:", maximum_us, "us")

else:

print("No valid responses; latency statistics unavailable.")

print("=============================")

run_latency_test()

This test measures how long the RP2040 takes to send a message to the ESP32 and receive its response. We’ll calculate the average, minimum, and maximum round-trip latency, along with successful responses and timeouts.