from machine import UART, Pin

import time

# RP2040 GPIO28 TX -> ESP32 GPIO16 RX

# RP2040 GPIO29 RX <- ESP32 GPIO17 TX

uart = UART(

0,

baudrate=115200,

bits=8,

parity=None,

stop=1,

tx=Pin(28),

rx=Pin(29)

)

TIMEOUT_MS = 2000

def crc16_ccitt(data):

# CRC-16/CCITT-FALSE

crc = 0xFFFF

for byte in data:

crc ^= byte << 8

for _ in range(8):

if crc & 0x8000:

crc = ((crc << 1) ^ 0x1021) & 0xFFFF

else:

crc = (crc << 1) & 0xFFFF

return crc

def read_line(timeout_ms=TIMEOUT_MS):

start = time.ticks_ms()

data = b""

while time.ticks_diff(time.ticks_ms(), start) < timeout_ms:

if uart.any():

chunk = uart.read()

if chunk:

data += chunk

if b"\n" in data:

return data.split(b"\n", 1)[0].strip()

time.sleep_ms(2)

return None

def make_frame(payload, corrupt_crc=False):

crc = crc16_ccitt(payload)

if corrupt_crc:

crc ^= 0x0001

frame = payload + b"|" + ("%04X" % crc).encode() + b"\n"

return frame, crc

def run_test(payload, corrupt_crc=False):

frame, crc = make_frame(payload, corrupt_crc)

# Expected FAIL for intentionally corrupted CRC;

# expected PASS for a valid CRC.

expected = b"FAIL" if corrupt_crc else b"PASS"

uart.write(frame)

response = read_line()

result = response == expected

print("Payload:", payload.decode())

print("CRC sent: %04X" % crc)

print("CRC intentionally corrupted:", corrupt_crc)

print("ESP32 response:", response)

print("Test result:", "PASS" if result else "FAIL")

print("--------------------------------")

return result

def run_crc_test():

# Clear stale input before testing.

while uart.any():

uart.read()

print("\n===== TEST 6: CRC-16 =====")

tests = [

(b"HELLO_ESP32", False),

(b"RP2040_DATA_12345", False),

(b"CORRUPTED_TEST", True)

]

passed = 0

for payload, corrupt in tests:

if run_test(payload, corrupt):

passed += 1

time.sleep_ms(100)

print("\n========== FINAL RESULTS ==========")

print("Total tests:", len(tests))

print("Passed:", passed)

print("Failed:", len(tests) - passed)

if passed == len(tests):

print("TEST 6: OVERALL PASS")

else:

print("TEST 6: OVERALL FAIL")

print("===================================")

run_crc_test()